access control custom API documentation

Version and revision records

| Version | describe | date |
| --- | --- | --- |
| V1.0 | create | 2019-06-06 _ _ _ _ |
| V1.1 | Modify Wiegand interface mode | 2019-06-25 _ _ _ _ _ |
| V1.2 | Add Wiegand input interface | 2019-08-09 |
| V1.3 | Add three-color lighting control | 2019-09-24 |
| V1.5 | The three-color light interface is compatible with multiple platforms Wiegand input adds active stop interface | 2019-12-26 |

Table of contents

1.	Version and revision records	1

2.	Table of contents	2

3.	How to use SMDT API in Android Studio	4

4.	SDK interface functions	5

4.1.	public int smdtSendCard(String idCard , int transformat)	5

Description: Wiegand interface operation, the parameter is ID card number .	5

4.2.	public int smdtSendCard(String HID_value, String PID_value, int transformat)	5

Description: Wiegand interface operation, the parameter is the public code hidden code .	5

4.3.	public String smdtReadWiegandData ( )	6

Description: Get Wiegand input, this is a blocking method, use the following parameters.	6

4.4.	public int smdtReleaseWiegandRead()	7

Description: Actively exit the blocking method smdtReadWiegandData(), which can only be used after version 1.5.	7

4.5.	public String[] smdtGetCameraVidPid(int cameraId)	7

Description: Read camera VID, PID interface	7

4.6.	public int setLedLighted(String ledColor, boolean lighted)	8

Description: Fill light operation (new API) -- white light, red light, green light . Only available after version 1.5	8

4.7.	public int s etControl(int type, int values)	8

Description: Fill light operation -- white light . (It is recommended to use setLedLighted for versions after 2019-12-26)	8

4.8.	public int smdtSetUsbPower(int type, int num, int values)	9

Description: Fill light operation -- red light . (It is recommended to use setLedLighted for versions after 2019-12-26)	9

4.9.	public int smdtSetGpioDirection(int gpioNumber, int direction, int value)	9

Description: Fill light operation -- green light . (It is recommended to use setLedLighted for versions after 2019-12-26)	9

4.10.	public int setRelayIoMode(int mode, int delay);	10

Description: Relay mode and delay settings .	10

4.11.	public int setRelayIoValue(int value);	11

Description: Turns the relay on or off .	11

4.12.	public int getRelayIoMode();	12

Description: In non-automatic mode, get the state of the relay	12

5.	For other general functions, please refer to the API documentation of the Seetech system_20180205.docx	错误!未定义书签。

How to use SMDT API in Android Studio

- Copy SmdtAccessControl_1_1.jar to [Project Directory\app\libs\];

2. Right-click the jar file in the libs folder and select add as Library...

Note: All api calls 
SmdtManager smdt = SmdtManager.create(this);

started with the SMDT API
  First declare the SmdtManager object, and then you can start using the API. For example: 
//declare the SmdtManager object 
private SmdtManager smdt ;

smdt = SmdtManager.create ( this ) ;

// 
use API smdt.setRelayIoValue ( 0 ) ;

SDK interface functions

public int smdtSendCard(String idCard , int transformat)

    - Description: Wiegand interface operation, the parameter is ID card number .

API version implementing this interface : V1.1

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| idCard | String | card number | 2147584137 |
| transformat | int | Format: Wiegand 26=1, Wiegand 34=2; | 1 |
| return value | int | success : 0, failure: -1 |  |

example:

SmdtManager smdt = SmdtManager.create ( this ) ;

smdt .smdtSendCard( "2147584137" , 1 ) ;

public int smdtSendCard(String HID_value, String PID_value, int transformat)

    - Description: Wiegand interface operation, the parameter is the public code hidden code .

API version implementing this interface : V1.0

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| HID_value | String | cryptic code | 001 |
| PID_value | String | public code | 34953 |
| transformat | int | Format: Wiegand 26=1, Wiegand 34=2; | 1 |
| return value | int | success : 0, failure: -1 |  |

example:

SmdtManager smdt = SmdtManager.create ( this ) ;

smdt .smdtSendCard( "001" , "34953" , 1 ) ;

public String smdtReadWiegandData ( )

    - Description: Get Wiegand input, this is a blocking method, use the following parameters.

API version implementing this interface : V1.0

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| return value | String | Return Wiegand Card Number | Such as: 4017669612 |

Note: Blocking method, swiping the card once will trigger once, it is recommended to open a thread in the Service to monitor all the time

example:

private class ReadThread extends Thread {
    @Override
    public void run() {
        while (!isInterrupted()) {
            String result = smdt.smdtReadWiegandData();
            if (result == null || result.equals("")|| 		result.equals("0")) {
                Log.i(TAG, "result ==== null............");
                continue;
            }
            Log.i(TAG, "result =======" + result);
         }
    }
}

when using it:

mReadThread = new ReadThread() ; 
mReadThread .start () ;

end :

@Override 
protected void on Stop () {
    super .onPause() ; 
if ( mReadThread != null ) {
        mReadThread .interrupt() ; 
//Version 1.5 and later can use the following code to exit the blocking function in time smdtReadWiegandData ()
        smdt.smdtReleaseWiegandRead () ;
    } 
}

public int smdtReleaseWiegandRead()

    - Description: Actively exit the blocking method smdtReadWiegandData(), which can only be used after version 1.5.

API version implementing this interface : V1. 5

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| return value | int | success: 0 fail: -1 | Such as: 4017669612 |

public String[] smdtGetCameraVidPid(int cameraId)

    - Description: Read camera VID, PID interface

API version implementing this interface : V1.0

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| cameraId | int | Camera ID value | 0 |
| return value | String [] | string[0] means vid, string[1] means pid |  |

be careful:

example:

SmdtManager smdt = SmdtManager.create ( this ) ;

String[] results = smdt .smdtGetCameraVidPid( 0 ) ;

Result: results[0]=34b3 ,results[1]=53a9

public int setLedLighted(String ledColor, boolean lighted)

    - Description: Fill light operation (new API) -- white light, red light, green light . Only available after version 1.5

API version implementing this interface : V1. 5

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| ledColor | String | SmdtManager.LED_WHITE white light SmdtManager.LED_RED red light SmdtManager.LED_GREEN green light |  |
| lighted | boolean | true: turn on the lights false: turn off the lights | 1 |
| return value | int | success : 0, Failed: -1 |  |

example:

SmdtManager smdt = SmdtManager.create ( this ) ;

smdt.setLedLighted ( SmdtManager.LED_WHITE , true ) ;

public int s etControl(int type, int values)

    - Description: Fill light operation -- white light . (It is recommended to use setLedLighted for versions after 2019-12-26)

API version implementing this interface : V1.0

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| type | int | fixed at 3 |  |
| values | int | 0: off, 1: on | 1 |
| return value | int | success : 0, failure: -1 |  |

example:

SmdtManager smdt = SmdtManager.create ( this ) ;

smdt.setControl ( 3 , _ _ 0 ) ;

public int smdtSetUsbPower(int type, int num, int values)

    - Description: Fill light operation -- red light . (It is recommended to use setLedLighted for versions after 2019-12-26)

API version implementing this interface : V1.0

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| type | int | fixed at 1 |  |
| num | int | Firmware is 3 |  |
| values | int | 0: off, 1: on | 1 |
| return value | int | success : 0, failure: -1 |  |

example:

SmdtManager smdt = SmdtManager.create ( this ) ;

smdt .smdtSetUsbPower( 1 , 3 , 1 ) ;

public int smdtSetGpioDirection(int gpioNumber, int direction, int value)

    - Description: Fill light operation -- green light . (It is recommended to use setLedLighted for versions after 2019-12-26)

API version implementing this interface : V1.0

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| gpioNumber | int | fixed at 4 |  |
| direction | int | fixed at 1 |  |
| values | int | 0: off, 1: on | 1 |
| return value | int | success : 0, failure: -1 |  |

example:

SmdtManager smdt = SmdtManager.create ( this ) ;

smdt .smdtSetGpioDirection( 4 , 1 , 1 ) ;

public int setRelayIoMode(int mode, int delay);

    - Description: Relay mode and delay settings .

API version implementing this interface : V1.1

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| mode | int | model: 0: Indicates no automatic closing mode, that is, it will not automatically close after opening the relay 1: Indicates automatic closing mode (high effective - default low level, then high level for X seconds, and finally low level), that is, after opening the relay, it will automatically close after a delay of X seconds 2: Indicates automatic closing mode (low effective - default low level, then high level for X seconds, and finally low level), that is, after opening the relay, it will automatically close after a delay of X seconds |  |
| delay | int | The delay of automatic closing mode, the unit is s, the maximum is 63s | 5 |
| return value | int | success : 0, failure: -1 |  |

example:

SmdtManager smdt = SmdtManager.create ( this ) ;

smdt .setRelayIoMode (1,5) ; //Auto mode, automatically close in five seconds

smdt .setRelayIoValue (1) ; //In automatic mode, both 0 and 1 can open the relay

public int setRelayIoValue(int value);

    - Description: Turns the relay on or off .

API version implementing this interface : V1.1

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| value | int | Non-electric closing mode, 0 means close the relay, 1 means open the relay. In auto-close mode, calling this method will open the relay. |  |
| delay | int | The delay of automatic closing mode, the unit is s, the maximum is 63s | 5 |
| return value | int | success : 0, failure: -1 |  |

example:

SmdtManager smdt = SmdtManager.create ( this ) ;

smdt .setRelayIoMode (0,5) ; //Non-automatic mode, the delay parameter is invalid

smdt .setRelayIoValue (1) ; //Open the relay

smdt .setRelayIoValue (0) ; //Close the relay

public int getRelayIoMode();

    - Description: In non-automatic mode, get the state of the relay

API version implementing this interface : V1.0

| parameter name / return value | type | illustrate | Example |
| --- | --- | --- | --- |
| return value | int | 0 means off state, 1 means on state |  |

example:

SmdtManager smdt = SmdtManager.create ( this ) ;

int status = smdt . getRelayIoMode (); //Get the status of the relay